SPRAYON PRODUCTS (PTY) LTD

MATERIAL SAFETY DATA SHEET

1. PRODUCT IDENTIFICATION AND COMPANY NAME
Product name:


Plastic Keycoat       Code: VP10

Product Description:




      
Company Identification:

Sprayon Products (Pty)Ltd





222 2nd Street






Wynberg Gauteng





South Africa

Emergency Contact No’s:
(2711) 440-2202


2. COMPOSITION / INFORMATION ON INGREDIENTS
Resin Base


: Chlorinated Oleoresin

Hazardous Components

: Alcohol/ Acetate type solvents
R Phases


: R11, 20/21, 34, 38, 39, 43, 45, 65

S Phases


: S1/2, 3,24/25, 26, 43, 45, 62

3. HAZARDOUS IDENTIFICATION
SAPMA Health Rating:

: 2-Moderate – Temporary or minor injury possible even 





  if treatment given

Physical / Chemical Hazards
: This paint is FLAMMABLE – see symbols below.

Human Health Hazards

: Respiration
: May cause irritation, dizziness, nausea







  and loss of consciousness.






: Eyes

: May cause moderate irritation. 






: Ingestion
: May cause gastro-intentional irritation, 







  Dizziness, nausea, and loss of 








  consciousness.






: Skin

: May cause sensitization. May also be a 






  skin irritant and repeated contact may 







  increase this effect.


Environmental Hazard

: Toxic to aquatic organisms. May cause short-term 





  adverse effects in the aquatic environment. Contain, 





  monitor and remove.


No carcinogenic, mutagenic, or genetic effects established.

 [image: image1.png]FLAMMABLE
LiQuID

By




                           [image: image2.jpg]HARMFUL




     [image: image3.jpg]


           [image: image4.png]



4. FIRST AID MEASURES
General

: In all cases of doubt or when symptoms persist, seek




  medical attention. Never give anything my mouth to an




  unconscious person.
Inhalation

: Move to a safe area with fresh air. Keep person warm and



  at rest. If not breathing, if breathing is irregular or if respiratory 



  arrest occurs, provide artificial respiration or oxygen by trained




  personnel. Give nothing by mouth, if unconscious, place in 



  recovery position and seek medical advice.



  attention.
Eye Contact

: Rinse thoroughly with potable water for at least 10 minutes.

Ingestion

: Do not induce vomiting. Keep person warm and at rest. Seek



  medical advice immediately and show this Safety Data Sheet




  to the medical officer.

Skin Contact

: Remove contaminated clothing and shoes. Wash skin 


  thoroughly with soap and water or use a recognized skin 

  cleanser. Do not use solvents or thinners.

5. FIRE FIGHTING MEASURES
Extinguishing Media
: Use foam, CO2, dry powders, and water mist. Do NOT use 



  a high pressure water jet.
Recommendations
: Decomposed products are a health hazard. Use appropriate 



  breathing apparatus. Cool closed containers exposed to fire 



  with water to prevent ignition.
6. ACCIDENT RELEASE MEASURES
Personal Precautions
: Ventilate the area. Avoid breathing vapors. Remove all 




  Sources of ignition e.g. flames, sparks static electricity and



  heat.
Spill


: Contain and collect spillage with absorbent material. Dispose of 




  according to local regulations. Do not allow spillage to enter




  sewerage, drains or watercourses. 
7. HANDLING AND STORAGE
Handling

: Keep container tightly closed. Eating, drinking, and smoking  
should be prohibited in area where this material is handled,         stored, and processed. After Handling the material, workers should wash hands and face before eating, drinking and

  smoking. Always comply with health and safety regulations.


Storage


: Store in original container. Observe label precautions. Store in 





  a cool, well ventilated area away from incompatible materials 




  and ignition sources. Keep away from oxidizing agents, strong



               alkaline and acids. NO smoking. Containers which have been 
                                         opened must be carefully resealed and kept upright to prevent 
  leakage. 

8. EXPOSURE CONTROL AND PERSONAL PROTECTION
Engineering Measures

: Provide adequate ventilation. Where reasonable 

  


  practicable, this should be achieved by the use of 




  local exhaust ventilation and good extraction. If these





  are not sufficient to maintain, concentrations of 





  particulates and solvent vapor below the OEL, 

  suitable respiratory protection must be worn.


Personal Protective Equipment


Respiration


: Should ventilation be inadequate or if workers are






  exposed to concentrations above the exposure limit,






  appropriate certified respirators must be used.


Eyes



: Use safety eyewear designed to protect against splash





  Of liquids. Never touch eyes with dirty hands or gloves.


Ingestion


: Observe the ruses of hygiene. Wash hands and face 





  before eating, drinking or smoking.


Skin



: Use protective gloves. Wash immediately in case of 





  contact. 

Environmental Exposure Control

: Do not allow product to enter sewerage drains or water-






  courses.

                                  [image: image5.png]


          [image: image6.png]


          [image: image7.png]\&v/




9. PHYSICAL AND CHEMICAL PROPERTIES
Appearance


: Thin Clear Liquid

Flash Point (°C)


: Below 21°C 

Solubility


: Alcohols / Acetates
10. STABILITY AND REACTIVITY
Stability



: Stable under recommended storage and handling




  conditions.
Conditions to Avoid

: Ignition sources, flames, sparks, static and heat.

Materials to Avoid

: Oxidizing agents, strong alkaline or acidic materials.

11. TOXICOLOGICAL INFORMATION
· Frequent and prolonged skin contact may cause irritation and skin inflammation.

· Prolonged inhalation of vapours in high concentrations may lead to headache, dizziness, and nausea.
· In case of contact with product, danger of absorption through the skin, irritation of the skin/mucous membranes.
12. ECOLOGICAL INFORMATION
· There is no data available on the preparation itself.
· Product considered a water pollutant. Do not allow to enter drains or watercourses.

13. DISPOSAL CONSEDRATIONS
· Do not allow to enter drains or watercourses.

· Dispose according to state and local applicable regulations.

· Empty containers are suitable for recycling.

14. TRANSPORT INFORMATION
SIN / UN No


: 1263

Shipping Name


: Paint Product


Packing Group


: II


Class



: 3.2
15. REGULATORY INFORMATION
Classification


: F, Xn

R11



: Highly Flammable

R20/21



: Harmful by inhalation and contact with the skin.

R34



: Causes burns.

R38



: Irritating to the skin.

R39



: Danger of very serious irreversible effects.

R43



: May cause sensitization by skin contact.

R45



: May cause cancer.

R65



: Harmful: May cause lung damage if swallowed.

S1/2



: Keep locked up/out of reach of children.

S13



: Keep away from food, drink, and animal feeding feeds.

S24/25



: Avoid contact with skin/eyes.

S26



: In case of contact with eyes, rinse immediately with 

  Plenty water and seek medical advice.


S43



: In case of fire, use water, fog, foam or powder – do not






  use water jets.


S45



: In case of accident or if you feel unwell, seek medical 

  advice immediately (show the label where possible).

S62



: If swallowed, do not induce vomiting: seek medical 





  advice immediately.

16. OTHER INFORMATION
Date of printing

: January 2020

Date of issue

: January 2017
Version


: 1
Notice


: The information in this MSDS is based on the present state of 
  our knowledge and on current national laws. The product is not 
  to be used for other purpose other than those specified under    

  section 1 without first obtaining written handling instructions. It 
  is always the responsibility of the user to take all necessary 
  steps in order to fulfil the demand laid down in the local rules 
  and legislation. The information on this MSDS is meant as a 
  description of the safety requirements of our product – it is not 
  to be considered as a guarantee of the products properties.
  No responsibility is accepted for errors or omissions or the 

  consequences thereof.



Release: 0722E
