SPRAYON PRODUCTS (PTY) LTD

MATERIAL SAFETY DATA SHEET

1.    PRODUCT AND COMPANY IDENTIFICATION

   Product name: 

Spot Putty (SP1)
   Company identification:  
Sprayon Products
222 2nd Street,

Wynberg Gauteng

   Emergency contact no: 
011-440-2202

   Email Address:

keithb@sprayon.co.za

      
 INCLUDEPICTURE "https://static.vecteezy.com/system/resources/previews/002/261/181/original/flammable-liquid-symbol-sign-isolate-on-white-background-illustration-eps-10-free-vector.jpg" \* MERGEFORMATINET 





2.    COMPOSITION AND INFORMATION ON INGREDIENTS
EC Classification:

F R11,66,67 S16,43A,51 

Hazardous and/or other relevant components

XYLENE <12.5%


[CAS #: 1330-20-7] [E.C. #: 601-022-00-9]

EEC Labels: XN OEL 435mg   OEL 100ppm STOEL 650mg    

STOEL 150.ppm RL+S 

N-BUTANOL <5%

[CAS #: 71-36-3] [E.C. #: 603-004-00-6

EEC Labels: F XN OEL 188
MG OEL 50ppm STOEL 560mg  

STOEL 150ppm RL+S

TOLUENE <12.5%

[CAS # 108-88-3] [E.C. @ 601-021-00-3]

EEC Labels: F XN OEL 188mg OEL 50ppm STOEL 560mg

STOEL 150ppm RL+S

2-BUTOXYETHANOL <12.5%

[CAS #: 111-76-2] [E.C.#: 603-014-00-0]

EEC Labels: XN OEL 120mg OEL 25ppm CL S

(CL/RL = Controlled / Recommended level ‘S’= Skin annotation)
3.    HAZARDS IDENTIFICATION
    
      SAPMA Health Rating: 3 –Slight irritation or minor reversible injury possible
                   Inhalation: Irritating
      Skin: Irritating risk of dermatitis. May be absorbed.
                   Eyes: Irritating

                   Ingestion: Irritating and nauseating
No carcinogenic, mutagenic or genetic effects established.
May have short term environmental effects. Contain, collect & remove.
4.    FIRST-AID MEASURES
Inhalation: Move to fresh air.
Skin: Use hand cleaner/soap & water. Remove contaminated clothing. If any discomfort get medical attention.
Eyes: Rinse immediately with plenty of water. Seek medical advice if symptoms persist.
Ingestion: Wash mouth with plenty of water. Do NOT induce vomiting. Seek        medical attention.

5.   FIRE-FIGHTING MEASURES
      Highly flammable with toxic fumes. 
      Containers can burst in a fire. 
      Can form explosive vapour/air mixture. Static discharge hazard!
      Foam. Dry powder. Fog to cool and control. Do NOT use water jets. 

      Cool containers in case of fire.
6.    ACCIDENTAL RELEASE MEASURES
Contain & collect. Use inert - NOT sawdust. Keep out of drains & sewers.
7.   HANDLING AND STORAGE
      Store only in designated places

      No open flames. No smoking. No sparks. Keep containers cool. 
      Avoid free fall of liquid – use earthing. 
      Subject to local bylaws.
8.   EXPOSURE CONTROLS/PERSONAL PROTECTION
      SAPMA Rating: 
3-H-E PPE: 
Respirator and Eye protection.

      Inhalation:
In case of insufficient ventilation, use suitable respiratory        protection.


      Skin:
Use barrier cream and impervious gloves.

      Eyes:
Use face shield & goggles or full face respirator/hood. 


Avoid direct contact.

      Ingestion:
Observe the rules for hygiene. Wash before eating, drinking


Or smoking.

      OEL Type:
Mixture-Mixture – See section 2

[image: image2.png]


 [image: image3.png]


 [image: image4.jpg]



9.    PHYSICAL AND CHEMICAL PROPERTIES
                   Chemical class:
PAINT

      Description:

THICK PASTE
      Smell:

Pungent

      VIC:

Paste SG:1.5  pH: t.b.e  NV: 70%
                   Boiling Point:  
>35 °C 
                   Flash Point: 
>23 °C  (derived)
10. STABILITY AND REACTIVITY
Stable under normal conditions.

Prone to ignite by static

Inert – no reaction with fire-fighting water.

11. TOXICOLOGICAL INFORMATION
No carcinogenic mutagenic or genetic effects established.

LD50 Oral: >500mg (derived)
12. ECOLOGICAL INFORMATION
May have short term environmental effects. Contain, collect and remove. 
13. DISPOSAL CONSIDERATIONS
Use reputable waste contractors. Exercise caution in disposal of used containers. 

14. TRANSPORT INFORMATION
      UN No: 1263 : PAINT PRODUCT
      Packing group: II IMO Class: 3.2

      EAC: 127 Hazchem: 3 [Y]E Kemler: 30



                     
[image: image5.jpg]FLAMMABLE
LIQUID




15. REGULATORY INFORMATION
EC No: n/a

EC Classification: F R11,66,67 S16,43A,51

R11 – Highly flammable

R66 – Repeated exposure may cause skin drying or cracking.

R67 – Vapours may cause drowsiness and dizziness.
S16 – Keep away from sources of ignition – No smoking.

S43A – In case of fire, use water fog, foam or powder – Do not use water jets!

S51 – Use only in well ventilated areas.


                                [image: image6.jpg]FLAMMABLE

shutterstock.com - 791628139




16. OTHER INFORMATION

TAKE PRECAUTIONARY MEASURES AGAINST STATIC DISCHARGES. 

IN CASE OF ANY DISCOMFORT ALWAYS SEEK MEDICAL ADVICE.

This safety data sheet provides the best advice under our current state of     knowledge. The user is ultimately responsible for ensuring that the requirements of the relevant legislation are complied with.

Date: 11/09/2024
