SPRAYON PRODUCTS (PTY) LTD

MATERIAL SAFETY DATA SHEET

1. Product and Company Identification

Product name:


Sprayon Electro Cleaner     Code: CCP40

Product Description:

Cleaner of all electrical equipment
Company Identification:

Sprayon Products (Pty)Ltd





222 2nd Street






Wynberg Gauteng





South Africa

Emergency Contact No’s:
(2711) 440-2202


2. Composition and Information on Ingredients


Name


Cas#

% by weight
Exposure Limits
N Propanol           
71-23-8
 
88.8

ACGIH(1996).







TWA:200ppm








STEL:250ppm










INHALATION Respirable








OSHA (PEL) [1994]








TWA:200ppm








STEL:250ppm








INHALATION Respirable








TWA:150ppm








TWA:450mg/M3

Sec-Butyl alcohol
78-92-2

+-12
3. Hazards Identification

Physical state and appearance:
Liquid

Emergency overview:


Causes eye irritation. May cause skin 







irritation. Maybe harmful if swallowed.

Routes of entry:


Eye contact. Ingestion. Inhalation. Absorbed 





through the skin.
Potential acute health effects:

Eyes:




Very hazardous in case of eye contact (irritant). 






Inflammation of the eye is characterized by 






redness, watering, itching: Sensitization of the 






product: Not available.


Skin:




Hazardous in case of skin contact (irritant 






permeator). Severe over exposure can result in 






death. Skin inflammation is characterized by 






itching, scaling, reddening or occasionally 






blistering 

Inhalation:



Extremely hazardous in case of inhalation 





(lung irritant). Maybe fatal if inhaled.

Ingestion:                                                Very hazardous in case of ingestion. Maybe 





fatal if swallowed.
Potential chronic health effects:
Carcinogenic Effects: Classified none by OSHA, 





none by NIOSH [n-Propanol]. Classified 4 





(probably not for human) by LARC [sec-Butyl 





alcohol] 






Mutagenic Effects: Classified suspected for 





human[n-Propanol]. Non-mutagenic for 






mammalians [n-Propanol]. Mutagenic for 





bacteria and/or yeast [n-Proponal].






Teratogenic Effects: Classified proven for 





human [n-Propanol]
Medical conditions aggravated by

Overexposure:



Persons with pre-existing eye, skin, respiratory, 





neurological or allergic conditions maybe more 





sensitive.






Persons also exposed to ethanol might be less 






sensitive.

Over exposure/signs/symptoms:
CNS depression: Dizziness, 








headache, incoordination, confusion, 







stupor,hematemesis,diarrhea, circulatory 






collapse, persistent coma, death by respiratory 






arrest.
4. First Aid Measures

Eyes:
Check for and remove any contact lenses. Immediately flush the eyes with running water for at least 15 minutes, keep eyelids open. Cold water may be used. DO NOT use eye ointment. Get medical attention.
Skin:
In case of contact, flush the skin with plenty of water. Gently and thoroughly wash the contaminated skin with running water and non-abrasive soap. Be particularly careful to clean folds. Cervices, creases and groin. Cold water may be used. Cover the irritated skin with an emollient. Wash clothes before reuse. Thoroughly clean shoes before reuse. If irritation persists, seek medical attention.                               Hazardous skin contact: Wash with disinfectant soap and cover skin with an anti-bacterial cream. Seek medical attention.
Inhalation:
Remove the victim from the source of exposure, and rest in half upright position and seek medical attention if irritation persists.

Ingestion:
Do not induce vomiting and seek medical attention if irritation.

5. Fire-Fighting Measures

Flammability of the product:
Flammable
Autoignition temperature:
400 deg
Flash Points:


Closed Cup: 25 deg  
Flammable Limits:

Lower: 2.1% Upper:13.5% 

Products of combustion: 
These products are carbon oxides.
Fire hazards inn presence of

Various substances:

Highly flammable in the presence of open flames and 





sparks, of het and oxidizing materials.


Explosion hazards in presence of 

Various substances:


Risk of explosion of the product in presence of 





mechanical impact

Firefighting media and instructions:   Flammable liquid, soluble or dispersed in 





water.






SMALL FIRE: use dry chemical CO2 alcohol 





foam or water spray.






LARGE FIRE: Use alcohol foam, water spray or 





fog. Cool containing vessels with water jet in 





order to prevent pressure build up. 

Protective Equipment:


Wear self-contained respirator and full 







protective gear.









 

6. Accidental Release Measures

Small Spill or leak:


Dilute with water and mop up or absorb with 






inert dry material and place in an appropriate 






water disposal. 
Large Spill or leak:


Flammable liquid. Keep away from heat. Keep 






away from sources of ignition. Stop leak if 






without risk. Absorb with dry earth, sand or other 





non-combustible material. Seal off all sewer 






exits. 

7. Handling and Storage
Handling:



Keep locked up. Keep away from heat. Keep 






away from sources of ignition. Do not ingest. 






Do not breathe gas, fumes, vapour or spray. 
Storage:



Flammable materials should be stored in a 






separate safety storage cabinet or room. Keep 






away from heat. Keep away from all sources of 






ignition. Keep container tightly closed. Keep in 






well ventilated area. A refrigerated room would 






be preferable for materials with a flash point 






lower than 37.8% cent.
8. Exposure Control/Personal Protection
Engineering Controls: 


Provide exhaust ventilation or other engineering 





controls to keep the airborne concentrations of 





vapour below their respective threshold limit 





values. Ensure that eyewash stations and safety 




showers are proximal to the work-station 





location. 
Inhalation: 



Vapour respirator. Wear appropriate respirator 





when ventilation is inadequate.

Skin: 




Lab Coat
Eyes: 




Splash goggles
Ingestion: 



Always observe rules of hygiene. Wash hands 





before eating, drinking or smoking.

Hand: 




Butyl rubber gloves.
General: 


Appropriate hand protection and protective 





clothing should always be used and suitable 





ventilation to be used at workstations with local 




             exhaust or breathing protection. Avoid prolonged or 




repeated exposure.
9. Physical and Chemical Properties
Physical State:


Liquid
Colour: 


Colourless Clear
Form and Odour 

Mild, alcohol like
Taste:



Not available 
Molecular weight/formula:
Not available
PH



Neutral
Boiling/condensation Points:
95deg C 
Melting/freezing Points:
-127deg C, 
Critical temperature:

The lowest known value is 263.7deg C. 

Specific gravity:

0.804(water=1)
Vapour pressure:

10mm of Hg (@20deg C.
Vapour Density:

2.1 (air =1)

Volatility:


100%vlv. [n-propanol]. 100(w/w) [n-propanol]

Odor threshold:

The highest known value is 30 ppm [n-propanol]. 





Weighted average 27.14ppm

Evaporation rate:

1.176 [Butyl acetate]

VOC:



100%
Viscosity:


The highest known value is 2.256 cP [n-propanol]

Solubility:


Easily soluble in cold water, hot water methanol, diethyl 





ether.

10. Stability and Reactivity
Stability: 


Stable if stored under normal conditions. 
Incompatibility with various 


substances;


Highly reactive with oxidizing agents.



 
11. Toxilogical Information
Toxicity to animals:

Warning: the LC50 values hereunder are estimated on 




the basis of a 4-hour exposure: Acute oral toxicity 




(LD50): 1870mg/kg [rat] (n-propanol). Acute toxicity of 




the vapour (LC50): 4000ppm 4 hours [rat] 9n-propanol). 




Acute dermal toxicity (LD50) 4060mg/kg [rabbit][ (n-




propanol)                                                                Chronic effects on humans:    Carcinogenic Effects: None                                         




Mutagenic effects: Suspected for human [n-propanol] 




Non-mutagenic for mammalians [n-propanol] Mutagenic 




for bacteria and or yeast[n-propanol]                       




Teratogenic effects: Proven for humans [n-propanol]      



Development toxicity: Reproductive 






system/toxin/female, reproductive system/toxin/male 




[suspected]  [n-propanol]    
Other toxic effects on human: Very hazardous in the case of eye contact.           




 Hazardous in case of skin contact, or inhalation or  I




 ingestion.

                                                              






 Special remarks on humans:   Can cause gastrointestinal disturbances. Exposure can 




 cause nausea, headache and vomiting. 

12. Ecological Information

Ecotoxicity:



Not available
BOD5 and COD:


No data available

Biodegradable/OECD:


No data available

Mobility:



No data available – possibly hazardous short-





term degradation products likely. However long-





term degradation products may rise.

Toxicity of the products of                   



                     biodegradation:


The products of degradation are less toxic 





than the product itself.                                 Special remarks on the products 





           of biodegradation:


No food chain concentration potential.

13. Disposal Considerations

General Information 


Do not puncture or incinerate, even when empty.

Disposal Methods

Dispose of waste to licensed waste disposal site in accordance with the requirements of the local waste disposal management. Containers should be thoroughly emptied before disposal because of risk of an explosion. Empty containers must not be punctured or incinerated because of the risk of an explosion.
14. Transport Information



14.1. UN number 
UN No. (ADR/RID)
1950






UN No. (IMDG
)
1950






UN No. (ICAO)

1950
14.2. UN Proper Shipping Name
Proper Shipping Name Aerosols






(ADR/RID)






Proper Shipping Name Aerosols






(IMDG)






Proper Shipping Name Aerosols






(ICAO)







Proper Shipping Name Aerosols






(ADN)

14.3. Transport Hazard Classes


ADR/RID Class

2.1

ADR/RID Label

2.1

IMDG Class

2.1

ICAO Class/Division
2.1

14.4. Packaging Group

         Not Applicable

14.5.Enviromental Hazards

 No
15   Regulatory Information
       Hazard symbol: Xn : Harmful by inhalation or if swallowed.

       Risk phrases:

       R20 : Harmful by inhalation.

       R21 : Harmful in contact with skin.

 
       R22 : Harmful if swallowed.

       Safety phrases:

       S24/25 : Avoid contact with skin and eyes.

       S2 : Keep out of reach of children.

16. OTHER INFORMATION

This safety data sheet provides the best advice under our current state of     knowledge. The user is ultimately responsible for ensuring that the requirements of the relevant legislation are complied with.

Date: 31/10/2025
Rev:  2025D
