SPRAYON PRODUCTS (PTY) LTD

SAFETY DATA SHEET
1. PRODUCT AND COMPANY INDENTIFICATION
Product name:


Polyurethane Alkyk 55% - Single Pack

Description:


Alkyd Resin

Product Code:


POL1 to POL8

Company Identification:

Sprayon Products (Pty)Ltd





222 2nd Street






Wynberg Gauteng





South Africa

Emergency Contact No’s:
(2711) 440-2202


2. HAZARDS IDENTIFICATION
2.1 Classification of the substance or mixture

[image: image1.jpg]



             STOT RE 1 H372

Causes damage to organs through prolonged or 3


Repeated exposure.

             Aso. Tox. 1 H304

May be fatal if swallowed and enters airways.

2.2 Label Elements

The substance is classified and labelled according to the Globally Harmonised System (GHS)

             [image: image2.png]



[image: image3.png]®
(&7




[image: image4.png]



                   GHS02
        GHS08
        GHS07

Hazard statements:
H226
Flammable liquid & vapour.

H302
Harmful if swallowed.

H312
Harmful in contact of skin.

H332
Harmful if inhaled.

H315
Causes skin irritation.

Precautionary statement (Prevention):

P280
Wear protective gloves/protective clothing/eye/face


Protection.

P210
Keep away from heat, hot surfaces, sparks, open flame


and other ignition sources. 


No smoking.

P202
Do not handle until all safety precautions have been 


read and understood.

P240
Ground/bond container and receiving equipment.

P242
Use only non-sparkling tools.

P243
Take precautionary measures against static discharge.


P241
Use explosion proof electrical/ventilating/lighting/


equipment.

P264
Wash with plenty of water and soap thoroughly after


handling.

Precautionary statement (Response):
P308 + P311
IF exposed or concerned: call a doctor/physician.

P305 + P351 + P338
IF IN EYES: Rinse cautiously with water for several 


minutes. Remove contact lenses, if present and easy to 


do. Continue rinsing.

P304 + P340
IF INHALED: Remove person to fresh air and keep 


comfortable for breathing.
P303 + P361 + P353
IF ON SKIN: Take off immediately all contaminated 


clothing, Rinse skin with water / shower. 

P370 + P378
IN case of FIRE: Use FOAM or dry powder to extinction.

Precautionary statement (Storage):
P403 + P235
Store in well-ventilated place. Keep cool.
P233
Keep container tightly closed.

Precautionary statement (Storage):

P501
Dispose of contents/container to hazardous or special


waster collection sites.

2.3 Other hazards
No specific dangers known, if the regulations / notes for storage and handling are considered.
3. COMPOSITION
3.1 Substances

Not applicable



3.2 Mixtures

Chemical nature of the mixture.

Preparation based on ALKYD RESIN modified with TDI:

Hazardous ingredients (GHS)

According to Regulation (EC) NO. 1272/2008
White Spirits

Content (w/w): >40% - <45%
Flam. Liq. 3

CAS Number: 64742-82-1

Asp. Tox. 1




STOT RE 1

Xylene (mix)

Content (w/w): >30% - >5%
Flam Liq. 3 (H226)

CAS Number: 1330-20-7

Acute Tox. 1 (H304)

EC-Number: 215-535-7

Acute Tox. 4 (H302)





Skin Irrit. 2 H315)





Eye Dam. 2 H318)





STOT SE 3 (H335)

1. Mixed Isomer

Content (w/w): 75% - 80%
Flam Liq. 3 (H226)

CAS Number: 1330-20-7
Acute Tox. 1 (H304)

EC-Number: 215-535-7
Acute Tox. 4 (H302)





Skin Irrit. 2 (H315)





Eye Dam. 2 (H318)





STOT SE 3 (H335)

2. Ethylbenzene

Content (w/w): 25% - >30%
Flam Liq. 3 (H225)

CAS Number: 100-41-4
Acute Tox. 4 (H332)

EC-Number: 202-849-4

4. FIRST AID MEASURES
Inhalation:


Remove patient to fresh air, keep warm and at rest.  If 






Recovery is not rapid, seek immediate medical attention.

Skin Contact:


Clean with a proprietary cleanser and wash with soap 





and water, do not use solvents. Remove contaminated




clothing and clean before re-use. If symptoms occurs, 





seek medical attention.

Eye Contact:


Flush eyes with plenty of water. If irritation persists, seek





medical attention.

Ingestion:


Do not induce vomiting. Rinse mouth with water. Seek





immediate medical attention.

5. FIRE FIGHTING MEASURES
Extinguishing Media:

Carbon dioxide, dry powder of foam; do not use water




except for cooling sealed containers.

Special Hazards:

May explode when heated or exposed to flames or 





Sparks. Combustion may produce hazardous 




Decomposition products.

Protective Equipment:

Self-contained breathing apparatus is recommended.

6. ACCIDENTAL RELEASE MEASURES
Personal Precautions:

Eliminate sources of ignition. Ensure adequate 





ventilation ensuring that the OEL is not exceeded





or use respiratory protection. Prevent skin and eye 





contact.

Environmental Precautions

Cleaning:


Prevent entry to drains, sewers or watercourses.





Contains spillage with sand,earth or approved absorbent





Place in suitable container, using spark proof equipment 





and dispose of in accordance with local regulations.

7. HANDLING AND STORAGE
Handling:


Ensure storage vessels and transfer equipment are 





earthed. Naked flames and other sources of ignition 





must not be present. Handle in well-ventilated areas.

Storage:


Suitable materials for storage are mild steel, stainless





steel and aluminum containers. Plastics and rubbers are





not recommended, although some polyolefins can be 





used for small quantities. Drums should be stored at 





ambient temperature. Can be stored outside well away 





from workplace or in flammable stores.

8. EXPOSURE CONTROLS / PERSONAL PROTECTION
ENSURE ADEQUATE VENTILATION.
Occupations Exposure

Limit (OEL):


8hrs TWA: 100ppm

Properties of solvent

10min STEL: 150ppm

Respiratory Protection:

Normal-working conditions should have adequate 





ventilation.

Hand Protection:
Wear PVC, nitrile, polyurethane or other suitable material gloves resistant to solvent.

Skin Protection:


Wear suitable protective clothing. Remove contaminated





clothing to avoid skin contact.

Eye Protection:  

Wear chemical splash goggles or safety glasses with 





side protection.

9. PHYSICAL AND CHEMICAL PROPERTIES
Appearance:



Pale Yellow Liquid
Odour:




With characteristic odour – solvent like.

pH:




Not determined

Boiling Point:



Typical 140 – 160 °C

Melting Point:



Not determined

Flash Point (°C)



Closed cup: 35°C

Evaporation Rate:


Not determined

Explosion/Flammability:


0.7 – 6.5% (V)

Auto Ignition Temperature:

296°C / 565°F

Vapour Pressure:


< 10mmHg @ 20°C

Density:



0.925 +- 0.005g/cm3

Solubility in Water:


Insoluble

Solubility in Solvent:


Soluble in most organic solvents

10. STABILITY AND REACTIVITY
Conditions to avoid:


Exposure to high temperatures of flames

Materials to avoid:


Strong oxidizing agents

Hazardous Decomposition

Products:



Burning will produce dense, acid smoke 






containing carbon dioxide, carbon monoxide






and other noxious fumes.









11. TOXICOLOGICAL INFORMATION
Inhalation:



High vapor irritates the eyes and the respiratory 






Tract, causing headaches, dizziness, nausea,






vertigo and unconsciousness and may have 






other nervous effects.

Ingestion:



Small amounts of liquid aspirated into the 






respiratory system during ingestion or vomiting






may cause broncho pneumonia or pulmonary 






edema. Low order of toxicity.

Skin contact:



Frequent or prolonged contact may irritate and 

cause dermatitis. May be absorbed with possible systemic effects

Eye contact:



Irritating

CMR Effects:



Carcinogenicity: Based on the available data, 

The substance is not suspected of having carcinogenic potential.

Mutagenicity: Results from in vitro and vivo tests do not lead to considering the product as genotoxic.

Reproductive toxicity: Based on the available data, the substance is not suspected of having reprotoxic potential.

12. ECOLOGICAL INFORMATION
Mobility:



Viscous Liquid
Persistence and Degradability:
Resin solids not readily biodegradable. Solvent vaporizes. 

Bioaccumulation:
Solvent can bioaccumulate but with short retention time.

Aquatic Toxicity:
Resin solids have lower order of toxicity. Solvent is moderately toxic to aquatic life.



Results of PBT and vPvB assessment: 
According to Annex X1111 of Regulation (EC) 


No. 1907/2006 concerning the Registration, Evaluation, Authorization and Restriction of Chemicals (REACH): The product does not contain a substance fulfiliing the PBT (persistent/bio accumative/toxic) criteria or the vPvB (very persistent/very bio accumative) criteria.

13. DISPOSAL CONSIDERATION

Liquid surplus of wastes should be appropriately labeled. Disposal must be in accordance with current national and local regulations. Appropriate disposal is by incineration. Solidifies wastes must be land filled at an appropriate H: H or H: h landfill site, but authorization from appropriate sources should be sought. Empty drums should be sent for reconditioning or scrap by a recognized drum reconditioner, appropriately labeled in accordance with national and local regulations. Empty containers pose a fire and explosion hazard. 
14. TRANSPORT INFORMATION
UN No.




1866

Hazchem Code:



3YE

UN Packaging Group:


111

RID/ADR/IMDG:


Class 3

UN Proper shipping Name

Resin Solution

IMDG Code:



Class 3

15. REGULATORY INFORMATION
· Safety, health and environmental regulations / legislation specific for the substance or mixture
If other regulatory requirements applies that is not already provided elsewhere in the SAFETY DATA SHEET, then it is described in the subsection.

· Chemical Safety Assessment

None
16. OTHER INFORMATION
Full text of the classification, including the hazard classes and hazard elements, if mentioned in Section 2 or 3:

Flam. Liq:



Flammable liquids

STOT RE 1:
Specific target organ toxicity (repeated exposure) – Category 1

Asp Tox 1:



Aspiration hazard – Category 1

PBT:




Persistent, Bioaccumulative & toxic

vPvB:




Very persistent and very bioaccumulative

Acute Tox:



Acute Toxicity

Skin Corr/Irrit:



Skin Corrission / Irritation

Eye Dam:



Serious eye damage

Skin Sens:



Skin Sensitization.

H304:




May be fatal if swallowed and enters airway

H372:




Causes damage to organs through prolonged or repeated exposure

H226:




Flammable liquid & vapour

H302:




Harmful if swallowed
H312:




Harmful in contact of skin

H332:




Harmful if inhaled

H315:




Causes skin irritation

H225:




Highly flammable liquid and vapour

H318: 




Causes serious eye damage

H319:




Causes serious eye irritation

H335:




May cause respiratory irritation

H373:




May cause damage to organs through prolonged or repeated exposure

To the best of our knowledge, the information contained herein is accurate. However, Sprayon Products (PTY) Limited does not assume any liability whatsoever for the accuracy or completeness of the information contained herein. Final determination of suitability of any material is the sole responsibility of the user. All materials may be present unknown hazards and should be used with caution. Although certain hazards are described herein, we cannot guarantee that these are the only hazards that exist.

Date: 140526
